APPENDIX  A

# HAZARDOUS MATERIALS IN THE DENTAL CLINICS

## General Precautions

- Handle chemicals properly in accordance with manufacturer instructions.

- Avoid skin contact with chemicals.

- Minimize chemical vapor in the air.

- Do not leave chemical bottles open.

- Do not use flame near flammable chemicals.

- Do not eat or smoke in areas where chemicals are used.

- When appropriate, wear protective eyewear and masks.

- Know proper cleanup procedures.

- Dispose of all hazardous chemicals in accordance with MSDS instructions and applicable local, state and federal regulations.

PhotographicChemicals  -  x-ray Fixer/Developer

Hazards:  Contact dermatitis, irritation of eyes, nose, throat and respiratory system from vapors and fine particulates of chemicals.

Do:  -  Use protective eyewear.

- Avoid skin contact with photographic chemicals and solutions by wearing  heavy –duty rubber gloves.

- Work in well ventilated areas.

- Clean up spilled chemicals immediately.

- Wash off chemicals with large amounts of water and a pH balanced soap, if contact occurs.

- Regularly launder clothing that comes in contact with photographic solutions.

- Store photographic solutions and chemicals in tightly covered containers.

Phosphoric  Acid -  Etching  Products

Examples:  Solutions and gels for acid etch techniques associated with placement of composites, sealants and orthodontic brackets usually contain phosphoric acid.

Hazards:  Acid burns and possibly sloughing of tissue, eye damage.

Do:  -Handle acid-soaked material with forceps or gloves.

-Avoid skin or soft tissue contact.

-Rinse with a large amount of running water in case of eye or skin contact.

Gypsum  Products – Model Stone / Plaster

Examples:  Gypsum products contain silica and calcium sulphate.

Hazards:  Irritation and impairment of respiratory system.

Silicosis.  Irritation of the eyes.

Do:  - Use plaster and other gypsum products in areas equipped with an exhust system.

-Use protective eyewear and NIOSH-approved mask while handling powders or

trimming models.

- Minimize exposure to powder during handling.

Mercury – Amalgum Capsules

Hazards:  Fine tremors, nausea, lossof appetite, diarrhea,  depression, fatigue,  increased irritability, allergic manifestations, contact dermatitis, pneumonitis, nephritis, headache, insomnia, dark pigmentation of marginal gingiva, loosening of teeth.

Do:  - Work in well-ventilated spaces.

- Avoid direct skin contact with mercury.

## Alcohol – Cleaning Solvent / Torch fuel

Hazards:  Fire, allergic manifestations, contact dermatitis, irritation to mucous membranes, respiratory problems, central nervous system depression, headache, drowsiness, loss of consciousness, nausea, liver and kidney damage, possible mutagenesis.

Do:  - Avoid skin contact.

- Avoid excessive inhalation of vapors.

- Avoid excessive inhalation of vapors.

- Work in well ventilated areas.

- Use forceps or gloves when handling contaminated gauze or brushes.

Phenolic Compounds -  Surface Disinfectant  (Omni  II)

Hazards:  Skin irritation / irritation of eyes, nose throat.

Do:  - Wear eye protection.

- Wear gloves.

- Use with adequate ventilation.

Formocresol – Pulpal Medicament

Hazards:  Skin irritation / irritation of eyes, nose throat.

Do:  - Wear eye protection.

- Wear gloves.

Glutaraldehyde – Cold Disinfection Solution

Hazards:  Eye contact may cause damage.  Skin contact irritation. Vapors may be

Irritating.

Do:  -  Wear eye protection.

- Wear gloves.

Butane – Lab Burner

Hazard:  Flammable

Do:  -  Wear gloves

-   Use with adequate ventilation.